Continues Assessment Sheet for Apprentice – Accredited Center (Company)

LEARN AND WORK ASIGNMENT (LWA)

| Sub Sector | ELECTRICAL POWER |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Training Occupation / Job Area | ELECTRICAL INSTALLATION & MAINTENANCE (SINGLE PHASE & THREE PHASE) (NOSS: EE-320-2/3: 2012) |  |  |  |  |  |  |  |  |  |  |  |  |
| Competency Unit Title | THREE PHASE DRAWING |  |  |  |  |  |  |  |  |  |  |  |  |
| Competency Unit Descriptor | Three phase drawing is covering the scope of competency to carry out three phase wiring activities for Three phase based on drawing given by utilizing three phase wiring tools, equipment and materials in compliance with Electrical Act 1990, Electrical Regulation 1994 and other related rules and regulations body and statutory requirements. The personnel who are competent in the three phase electrical drawing must be able to identify drawing specification for three phase three phase wiring and carry out drawing according to standard requirements. |  |  |  |  |  |  |  |  |  |  |  |  |
| Competency Unit Code |  | Competency Type |  | Core | Level |  | 3 |  | Duration | 72 |  | Credit Hours |  |
| Social Values & Social Skills: | Compliance - Conceptual skills - Learning skills |  | Self-reliance - Learning skills - Multitasking and prioritizing |  |  | Cooperation - Communication skills - Interpersonal skills - Teamwork |  |  |  |  | Punctuality - Leadership skills - Self-discipline |  |  |
| Tools/ Technologies, Equipment and Materials (TEM) |  |  |  |  |  |  |  | Ratio (TEM : Apprentices) |  |  |  |  |  |
| - Schematic diagram - Block Diagram - Measuring Instruments - Electrical components/Parts Specification - Equipment Data Book - Drawing instrument - Computer - Computer Added Drawing software |  |  |  |  |  |  |  | 1 : 1 1 : 1 1 : 1 1 : 1 1 : 2 1 : 1 1 : 1 1 : 5 |  |  |  |  |  |
| References/ Information Source |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Undang-Undang Malaysia. 2005. Akta Bekalan Elektrik 1990 dan Peraturan-Peraturan Elektrik 1994, Pindaan sehingga 2005. MDC Publisher Sdn Bhd Malaysia. - Suruhanjaya Tenaga Malaysia. Standard Pendawaian Malaysia (Electrical Installations Of Buildings - MS IEC 60364) Jabatan Standard Malaysia - The Institution of Electrical Engineers, 1998, Peraturan-Peraturan bagi Pemasangan Elektrik, Edisi 16. Golden Books Centre Sdn Bhd - Giesecke, F. E. et. Al. 2002. Technical Drawing. Prentice Hall - BS 7671: 2001 Requirements for Electrical Installations (IEE Wiring Regulations, Sixteenth Edition) Institution of Electrical Engineer. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Instructions, Guiding Questions & Hints Use the following instructions, guiding questions and hints in assisting you to complete the above assignment Setting Goal: Perform site visit to gather information according to the specification below: - Determine access site permission from (customer, owner) - Ensure follow customer/ owner procedure requirement for site visit - Interpret the project drawing (building layout plan): - View project specification - Identify electrical work specification - Type of wiring (surface/ conduit/ tranking) - Type of load (lamp, air-cond, power point) - Identify wiring Route - Measure the actual length for wiring route - Confirm position of point - Sketch layout plan based on gathered information B. Planning: Plan the drawing requirement - Acquire drawing set. - Identify Computer Added Drawing. - Identify computer system. - Identify drawing software. C. Decision Making: Decide the drawing requirement by present the work plan to coach before executing the assignment - Select drawing set. - Select Computer Added Drawing. - Select computer system. - Make sure use drawing software according to drawing design requirement. D. Executing and Monitoring: Produce three phase drawing using computer drawing software or manual drawing (based on company application) - Locate the point into the project drawing (building layout plan) for wiring. - Apply electrical symbol according to electric standard - Calculate total connected load (TCL) according to Electrical Regulations 1994 and MS IEC 60364 Standard (Local Standard). The calculation based on: - type of load - diversity factor - type premis - Calculate maximum demand (MD) for income power supply. - Requirement from power supply authority - Use maximum demand formula from Electrical Regulation - Draw and configure single line / schematic diagram drawing - Apply electrical single line wiring symbols - Mark value for each component symbols - Markcable andcable size - Mark quantity load for each final sub-circuit. - Mark TCL & MD at foot note drawing - Draw and configure three phase wiring drawing (Shop drawing) - Apply electrical single line wiring symbols - Draw electrical wiring route at the project drawing (building layout plan) - Mark wiring route distance - Mark cable, type, size and number of wiring per-route - Mark the components tagging - Draw and configure three phase motor control circuit. - Produce the final schematic and shop drawing. - Make sure portray the attitude listed below when performing this cycle’s activities. Attitude: - Handle computer with care - Adhere to ergonomic requirements - Meticulous in drawing - Meticulous in calculating TCL & MD - Adhere to safety regulation E. Evaluating: Check drawing specification against design requirements with your coach. - Ensure types of drawing produced - Assess value of TCL & MD listed - Check drawing error: - Symbols - Line connection - Labeling & marking (size cable, load, breaker) - Refine and Re-produce three phase schematic & shop drawing (if have any drawing error) - Submit for endorsement from authorize personel according to electrical regulation. - Carry out housekeeping activities. |  |  |  |  |  |  |  |  |  |  |  |  |  |