Continues Assessment Sheet for Apprentice – Accredited Center (Company)

LEARN AND WORK ASIGNMENT (LWA)

| Sub Sector | ELECTRICAL POWER |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Training Occupation / Job Area | ELECTRICAL INSTALLATION & MAINTENANCE (SINGLE PHASE & THREE PHASE) (NOSS: EE-320-2/3: 2012) |  |  |  |  |  |  |  |  |  |  |  |  |
| Competency Unit Title | THREE PHASE WIRING |  |  |  |  |  |  |  |  |  |  |  |  |
| Competency Unit Descriptor | Three phase wiring covering the scope of competency to carry out wiring activities for Three phase based on drawing given by utilizing wiring tools, equipment and materials in compliance with Electrical Act 1990, Electrical Regulation 1994 and other related rules and regulations body and statutory requirements. The personnel who are competent in the three phase electrical wiring must be able to interpret three phase wiring specifications and procedures, three phase wiring requirement, carry out wiring activities, earthing system installation and carry out continuity, polarity, insulation and resistance testing on the three phase wiring system. |  |  |  |  |  |  |  |  |  |  |  |  |
| Competency Unit Code |  | Competency Type |  | Core | Level |  | 3 |  | Duration | 192 |  | Credit Hours |  |
| Social Values & Social Skills: | Compliance - Conceptual skills - Learning skills |  | Self-reliance - Learning skills - Multitasking and prioritizing |  |  | Cooperation - Communication skills - Interpersonal skills - Teamwork |  |  |  |  | Punctuality - Leadership skills - Self-discipline |  |  |
| Tools/ Technologies, Equipment and Materials (TEM) |  |  |  |  |  |  |  | Ratio (TEM : Apprentices) |  |  |  |  |  |
| - Schematic diagram - Block Diagram - Measuring and Test Instruments - Components/Parts Specification - Distribution Boards - Trunking Tools - Equipment Data Book - Electrical Equipment - Electrical hand tools - Electrical power tools - Electrical Mechanical tools - Electrical Accessories - Electrical Fittings - Electrical Testing Instrument - Double Insulated cable - Wiring lead - Wiring neil - Cables - PVC Tape - Standard Operating Procedure |  |  |  |  |  |  |  | 1 : 1 1 : 1 1 : 1 1 : 1 1 : 2 1 : 1 1 : 1 1 : 1 1 : 1 1 : 1 1 : 1 1 : 1 1 : 1 1 : 1 1 : 1 1 : 1 1 : 1 1 : 1 1 : 1 1 : 1 |  |  |  |  |  |
| References/ Information Source |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Undang-Undang Malaysia. (2005). Akta Bekalan Elektrik 1990 dan Peraturan-Peraturan Elektrik 1994, Pindaan sehingga 2005. MDC Publisher Sdn Bhd Malaysia. - Suruhanjaya Tenaga Malaysia. Standard Pendawaian Malaysia (Electrical Installations Of Buildings - MS IEC 60364) Jabatan Standard Malaysia - The Institution of Electrical Engineers. (1998). Peraturan-Peraturan bagi Pemasangan Elektrik, Edisi 16. Golden Books Centre Sdn Bhd - Giesecke, F. E. et. Al. 2002. Technical Drawing. Prentice Hall - BS 7671: 2001 Requirements for Electrical Installations (IEE Wiring Regulations, Sixteenth Edition) Institution of Electrical Engineers |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Instructions, Guiding Questions & Hints Use the following instructions, guiding questions and hints in assisting you to complete the above assignment - Setting Goal: Identify three phase wiring specifications - Identify site location for wiring work - Acquire work permit from authority before access the site (Green Card-CIDB, safety passport-NIOSH) - Identify wiring work schedule and time frame by referring to immediate supervisor. - Determine location of the wiring route by referring to layout drawing - Determine types of wiring by referring to BQ. (either Conceal, conduit, surface) - Determine types of protection devices (MCB, ELCB, RCCB) by referring to BQ - Determine types of earthing system (TT, TN-C, TN-S, TN-C-S) by referring to Authorise electricity supplier. - Identify type of earth conductor by referring to BQ - Determine types of conductor (Cable, bus-bar) by referring to BQ - Determine suitable fitting and accessories to be installed by referring to schematic & layout drawing and BQ. - How many 3 phase power point(S/S/O)- for three phase? - How many 3 phase Water Heater point? - How many 3 phase Air-Cond points? - 3 phase motor & motor control - Prepare documentation requirement for supply application: - Schematic Drawing - Form G B. Planning: Plan three phase wiring tools, equipment and materials - Identify wiring tools, equipment and materials (TEM), such as: - Hand tools (player, cutter, hammer, screw driver, hand hacker) - Power tools (Drill, grinder, hacker, hole saw, ) - Tester device (E.g.: test pen, multi-meter) - Equipment (E.g.: spring/ conduit bender, bender machine, threading machine, reamer machine, ladder, trolley) - Identify wiring conductor specification by referring to schematic drawing and BQ according to: - size - quantity - colour code - type of cable (E.g.: Core and Insulation) - sequence of cable connection (Red, yellow Blue) - Identify Wiring Material and accessories (E.g.: Elbow, Tee Box, coupling socket, conduit) - Identify electrical fittings and accessories (E.g.: lamp, fan, air-Cond, heater) - Identify material and accessories for earthing system installation by referring to Schematic drawing and BQ - Identify material and accessories for DB installation by Referring to Schematic drawing and BQ - Identify testing tools and equipment to perform: - Insulation test - polarity test - Earth continuity test - Earth resistance test - Identify safety equipment - What PPE should be wear? - Ensure your work place have Fire extinguisher. - Make sure bring on site with first aids kits - Make sure bring on site with safety tag (barrier tape, sign board) - Identify logistic for the work: - Identify transportation - Identify storage location on the work site - Identify manpower requirement - How many personnel skills (e.g.: competence, electrician)? - How many helpers (general worker)? - Make sure portray the attitude and adhere to the safety listed below when performing this cycle’s activities. Attitude: - Handle wiring tools with care Safety: - Ensure wiring tools in good working condition C. Decision Making: Decide the drawing requirement by present the work plan to coach before executing the assignment - Select wiring tools, equipment and materials (TEM). - Confirm wiring conductor specification by referring to schematic drawing and BQ. - Select wiring material and accessories. - Select electrical fittings and accessories. - Select material and accessories for earthing system installation by referring to Schematic drawing and BQ - Select material and accessories for DB installation by Referring to Schematic drawing and BQ - Select testing tools and equipment. - Select safety equipment. - Confirm logistic for the work. - Confirm manpower requirement. D. Executing and Monitoring: Carry out wiring, DB and Earthing Installation - Are your work place required authorize permission before proceed the work? - Authorise person (E.g.: project/ site manager) - Premise owner - Setup safe working area based on: - Wear PPE - Tag working area - Are your work place required permits to work (PTW)? - Ensure follow the safety procedure in using the tools and equipment. Make sure all temporaries power supply should follow safety procedure. - Mark location point according to layout drawing by following the standard MS IEC 60364 - Wire route - DB/ consumer unit location - Accessories location (Switch and socket) - Motor & Motor starter location - Clear wiring route. - Cutting and Hacking if necessary - Follow safety procedure - Install wiring conduit and accessories based on marked location point. - Ensure following the procedure of installation process - Make good the effected wiring route area (E.g.: Re-plaster, repaint) - Install cable according to layout drawing. - Ensure the cable specification to be installed according to point function. - ensure the cable in good condition before, during and after the installation process - Make sure the earth conductor install according device/ circuit requirement. - Mark the cable according to point function and polarity (R/Y/B). - Confirm installation cable according to specification. - Apply visual inspection - Check polarity(RYB), condition and size of cable - Install electrical fittings and accessories according to device requirement - Ensure all the cable connection correctly (live-RYB, neutral and earth) - Perform earthing system installation according specification (MS IEC 60364) - Install electrode (copper rod/ GI pipe, clamp, earth conductor) - Install earth chamber - Connect earth continuity conductor - Install sub-main incoming supply cable - Ensure the cable specification to be installed according to schematic drawing. - ensure the cable in good condition before, during and after the installation process - Mark the cable according to point function and polarity (RYB). - Install three phase Distribution Board (DB) - Align DB box - Fix protection device - Fix internal connection wiring - Connect wiring cable to DB - Make sure tidiness of wiring cable - Terminate the wiring cable according to schematic drawingand wire mark - Connect earth continuity conductor - Make sure portray the attitude and adhere to the safety listed below when performing this cycle’s activities. Attitude: - Meticulous - Housekeeping - Ensure no wastage of cable - Handle testing equipment with care Safety: - Use appropriate PPE during carry out wiring activities - Safety tools & equipment in proper order - Evaluating: Evaluate the three phase wiring functionality with your coach. - Confirm all wiring work installation according to specification. - Apply visual inspection - Produce inspection report according to power supply authority requirement - Fulfill Form G - Ensure follow Electrical regulations - Carry out dead circuit test(Pre-test: without supply) according to power supply authority requirement - Perform Insulation test - Perform polarity test - Perform Earth continuity test - Perform Earth resistance test - Rectify / Coordinate rectification work for non-compliance test result - Produce Test report according to power supply authority requirement - Fulfill Form H- Ensure follow Electrical regulations - Prepare Test result - Prepare As-built schematic drawing (if necessary) - Submit the report (form G & H) to supervisory for approval. - Make sure portray the attitude and adhere to the safety listed below when performing this cycle’s activities. Attitude: - Housekeeping working area after wiring activities - Meticulous in writing report Safety: - Use appropriate PPE during carry out earthing & installation works - Safety tools & equipment in proper order |  |  |  |  |  |  |  |  |  |  |  |  |  |