Continues Assessment Sheet for Apprentice – Accredited Center (Company)

LEARN AND WORK ASIGNMENT (LWA)

| Sub Sector | ELECTRICAL POWER |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Training Occupation / Job Area | ELECTRICAL INSTALLATION & MAINTENANCE (SINGLE PHASE & THREE PHASE) (NOSS: EE-320-2/3: 2012) |  |  |  |  |  |  |  |  |  |  |  |  |
| Competency Unit Title | SINGLE PHASE WIRING TESTING AND COMMISSIONING |  |  |  |  |  |  |  |  |  |  |  |  |
| Competency Unit Descriptor | Single phase wiring testing & commissioning is a competency to carry out testing and inspection on Single Phase wiring system for the purpose of commissioning the wiring system according to Electrical Act 1990, Electrical Regulation 1994 and other related rules and regulations body and statutory requirements. The personnel who are competent in the single phase electrical wiring must be able to identify single phase testing and commissioning specifications and procedures, carry out electrical testing including resistance testing, polarity testing, continuity testing, earthing testing and carry out commissioning on the wiring system |  |  |  |  |  |  |  |  |  |  |  |  |
| Competency Unit Code |  | Competency Type |  | Core | Level |  | 2 |  | Duration | 100 |  | Credit Hours |  |
| Social Values & Social Skills: | Compliance - Conceptual skills - Learning skills |  | Self-reliance - Learning skills - Multitasking and prioritizing |  |  | Cooperation - Communication skills - Interpersonal skills - Teamwork |  |  |  |  | Punctuality - Leadership skills - Self-discipline |  |  |
| Tools/ Technologies, Equipment and Materials (TEM) |  |  |  |  |  |  |  | Ratio (TEM : Apprentices) |  |  |  |  |  |
| 1. Schematic diagram 2. Block Diagram 3. Measuring and Test Instruments 4. Components/Parts Specification 5. Equipment Data Book |  |  |  |  |  |  |  | 1:1 1:1 1:1 1:1 1:2 |  |  |  |  |  |
| References/ Information Source |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Joshi. (2008). Residential, Commercial and Industrial Electrical Systems: Protection, testing and commissioning. Tata McGraw-Hill Education. - Keith Harker. (1998). Power System Commissioning And Maintenance Practice. IET Limited - Undang-Undang Malaysia. 2005. Akta Bekalan Elektrik 1990 dan Peraturan-Peraturan Elektrik 1994, Pindaan sehingga 2005. MDC Publisher Sdn Bhd Malaysia. - Suruhanjaya Tenaga Malaysia. Standard Pendawaian Malaysia (Electrical Installations Of Buildings - MS IEC 60364) Jabatan Standard Malaysia - The Institution of Electrical Engineers, 1998, Peraturan-Peraturan bagi Pemasangan Elektrik, Edisi 16. Golden Books Centre Sdn Bhd |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Instructions, Guiding Questions & Hints Use the following instructions, guiding questions and hints in assisting you to complete the above assignment - Setting Goal: Identify single phase testing and commissioning specifications - Identify location to carry out the testing & commissioning - Interpret single phase schematic diagram for functional testing - Determine types of testing - Incoming Supply variation (Voltage Test) - RCD test - Functional test - Socket polarity test - Comply Safety regulation bodies - Determine energy supply approval and authorization permission to carry out the testing activities - Planning: Plan single phase testing and commissioning tools, equipment and materials - Determine preparation tools and equipment for testing and commissioning activities - Ensure the tools and equipment calibrated - Determine testing and commissioning format form for single phase requirement. - Identify electrical circuit in the single phase wiring by interpret the as-built schematic drawing 8.1 suit the testing according to circuit requirement (plug-socket polarity test) - Identify safety equipment - What PPE should be wear? - Ensure your work place have Fire extinguisher. - Make sure bring on site with first aids kits - Make sure bring on site with safety tag (barrier tape, sign board) - Identify manpower requirement - How many personnel skills (e.g.: competence, electrician)? - How many helpers (general worker)? - Make sure portray the attitude listed below when performing this cycle’s activities. Attitude: - Handle equipment with care - Decision Making: Prepare single phase testing and commissioning tools, equipment and materials by present the work plan to coach before executing the assignment - Select tools and equipment for testing and commissioning activities. - Select testing and commissioning format form for single phase requirement. - Confirm electrical circuit in the single phase wiring by interpret the as-built schematic drawing. - Select safety equipment. - Make sure manpower sufficient for the activities. - Executing and Monitoring: Perform single phase testing and commissioning according to MS IEC 60364 standard procedure - Realign the tools and equipment on site - Ensure the wiring circuit ready for connect to energy supply meter installed. - Perform Incoming Supply variation (Voltage Test)for wiring - Perform RCD test for wiring - Insulation test performed according to SOP - Perform Socket polarity test for wiring - Earth continuity conductor test performed according to SOP - Earth resistance test performed according to SOP - Perform electric motor installation test - Perform electrical appliances test - Make sure portray the attitude and adhere to the safety listed below when performing this cycle’s activities. Attitude: - Housekeeping - Handle equipment with care Safety: - Used appropriate PPE - Used tools, equipment and material properly - Evaluating: Evaluate single phase testing and commissioning activities with your coach. - Confirm testing result according to specification requirement. - Refer to MS IEC 60364 standard, code of practice, power supply authority. - Rectify / Coordinate rectification work for non-compliance test result. - Record the testing result in testing and commissioning form - Ensure the format according to company’s requirement - Prepare testing and commissioning report - Compile testing and commissioning result report - Form G & H - Test result - schematic drawing (latest) - Power supply application form - Submit testing and commissioning report to superior for endorsement by competence person for power supply application: - Submit electric motor testing and commissioning report to superior: - Test result - schematic drawing (latest) - Submit electric appliances testing and commissioning report to superior: - Test result - schematic drawing (latest) - Make sure portray the attitude and adhere to the safety listed below when performing this cycle’s activities. Attitude: - Handle testing equipment with care Attitude: - Meticulous in fill in commissioning form - Meticulous in preparing report - Adhere to report submission dateline |  |  |  |  |  |  |  |  |  |  |  |  |  |