| LEARN AND WORK ASIGNMENT (LWA) |  |  |  |
| --- | --- | --- | --- |
| NOSS (CODE NOSS) | MOTORCYCLE SERVICING AND MAINTENANCE (TP-118-1; 2012 |  |  |
| Competency Unit Title (CU CODE) | BRAKE SYSTEM SERVICING (ETM-XXX-05) | LEVEL | 1 |
| Competency Unit Descriptor | Brake system servicing is a set competency of skills to carry out service brake system that has been recognized by manufacturer’s manual requirements. The personnel whom are competent in the brake system servicing must be able to prepare brake system servicing requirements, perform brake system troubleshooting, brake pads/shoes servicing and replacement, brake fluid replacing, disk brake/brake drum servicing and replacement, brake hose/cable replacement, brake system functionality check and record brake system servicing. Brake system servicing is to ensure that motorcycles can stop safely when needed. |  |  |
| Candidate Name |  |  |  |
| Candidate I/C Number |  |  |  |
| Company’s Name |  |  |  |

CU WORK ACTIVITY STATEMENT:

PERFORM BRAKE SYSTEM SERVICING ACTIVITIES

DURATION: 46 HOURS

- SETTING GOAL :

You are required to perform brake system servicing activities based on performance criteria below:

  - Types of servicing for brake system determined

  - Servicing tools, equipment and materials prepared

  - Types of defect in brake system identified based on manufacturer’s specifications

  - Brake pads / shoes serviced / replaced according to manufacturer’s specification

  - Brake fluid replaced according to motorcycle manuals

  - Disk brake / brake drum faulty identified

  - Disk brake / brake drum serviced /  replaced according to the motorcycle manufacturer’s manual

  - Brake hose / cable replaced according to the motorcycle manufacturer’s manual

  - Brake system checks according to the motorcycle manufacturer’s manual

  - Service checklist updated

  - Service checklist submitted to superior

B. PLANNING

You are required to plan activities to achieve listed setting goal of performing brake system servicing activities by using resources listed below:

  - Identify  tools, materials and equipment for performing brake system servicing activities according to listed below:

| ITEMS | RATIO (TEM : Trainees) |
| --- | --- |
| - Brake system - Drum brakes - Disc brakes - Brake pad/brake shoes - Asbestos pad - Metal contains pad - Service manual - Air bleed vacuum - Brake pump - Brake caliper - Part tray - Torque wrench - Waste cloth/rag - Brake fluid - Hand tools : - Wrenches - Allen keys - PPE - White spirit/Water - Air compressor - Blower gun - Measuring tools - Surface plate - Feeler gauge - Dial indicator - Vernier caliper - Sand paper | 1 : 1 1 : 1 1 : 1 1 : 1 1 : 5 1 : 5 1 : 1 1 : 1 1 : 1 1 : 5 1 : 1 As per required 1 : 1 1 : 1 As per required 1 : 20 1 : 20 1 : 20 As per required |

  - You may refer to the references below as a guide to perform this activity.

REFERENCES

- Mark Zimmerman. (2004). The essential guide to motorcycle maintenance: Tips & Techniques to Keep Your Motorcycle in Top Condition. ISBN1884313418, 9781884313417. Whitehorse Press Series. - Dave Tharp. (1991)Trelland’s Motorcycle Reference Guide – Vol. 1. ISBN 1-895542-00-6. Trelland Industries Ltd - de Rome, L., Stanford, G. & Wood, B. (2007), Positioned for Safety 2010: A Motorcycle Safety Strategic Plan 2007-2010, Motorcycle Council of NSW, Inc., - Gibson T. & E. Benetatos (2000), Motorcycles and crash barriers prepared by Human Impact Engineering for the Barriers to Safety campaign, NSW Motorcycle Council, Sydney - Hurt, H.H. Jr., Ouellet, J.V. & Thom, D.R. (1981), Motorcycle Accident Cause Factors and Identification of Countermeasures, Final Report to National Highway Traffic Safety Administration, US Department of Transportation, PB 81-206443, 81-206450.

  - List out any references related  when performing this activity such as:

    - Standard Operating Manual (SOP)

    - Service manual / Manufacturer’s manual/ Workshop manual

    - Service job sheet

    - Owner’s manual

    - Etc.

- DESION MAKING

You are required to get coach approval before performing brake system servicing activities.

- EXECUTE & MONITORING

You are required to perform brake system servicing activities according to the steps below:

  - Prepare brake system servicing requirements

    - Determine types brake system for motorcycle

- Disc brake system

- Drum brake system

    - Interpret manufacturer’s brake system servicing manual

    - Select brake system servicing tools, equipments and materials

    - Obtain  service job sheet from service advisor (SA)

    - Determine types of brake system

  - Perform brake system service inspection

    - Refer job sheet from Service Advisor (SA) to identify faulty

  - Perform brake pads servicing

    - Remove brake pads from the brake calliper

    - Clean brake pads

    - Check brake pad thickness

- Observe fluid level at reservoir

- Observe wear limit of the brake pad

- Measure brake pad thickness

  - Replace brake pads according to service manual

    - Check leakage at brake caliper, brake master pump and brake hose according to service manual

- Replace if necessary

  - Perform brakehose replacement

  - Perform brake fluid refill

    - Determine types of brake fluid according to model of motorcycle such as:

- DOT 3

- DOT4

    - Check brake fluid level

    - Refill brake fluid according to procedure and service manual

  - Perform brake drum servicing

    - Remove  brake drum from the motorcycle

    - Clean brake shoes and drum

    - Check brake lining thickness

- observe wear limit of the brake lining

- Measure brake lining thickness

    - Replace brake shoes according to service manual

    - Assemble brake components to motorcycle according to procedure and service manual

    - Perform brake cable replacement

- Check cable wear and tear

- Lubricate the cable

    - Carry out brake shoes adjustment

  - Carry out test ride

    - Check brake system functionality according to service manual

    - Inform SA that motorcycle ready for a test ride

  - Record servicing activities

  - Submit record to SA

  - Carry out housekeeping activities such as:-

    - 5’S

  - Comply with attitude, safety and environment listed below when performing this activity

  - Apply core abilities listed below when performing this activity

| Social Skills | Core Abilities |
| --- | --- |
| Communication skills | 02.03 Communicate clearly |
| Conceptual skills | 01.01 Identify and gather information 02.01 Interpret and follow manuals, instructions and SOP’s |
| Interpersonal skills | 03.06 Respond appropriately to people and situations |
| Multitasking and prioritizing | 01.02 Document information, procedures or processes 02.04 Prepare brief reports and checklists using standard form |
| Self-discipline | 03.03 Accept responsibility for own work and work area 03.02 Demonstrate integrity and apply ethical practices |
| Teamwork | 03.01 Apply cultural requirements for the workplace |
| Learning skill | 06.01 Understand systems |
| Leadership skill | 03.06 Respond appropriately to people and situations 03.03 Accept responsibility for own work and work area |

- EVALUATING

You are required to evaluate brake system servicing activities using the checklist below.

| A | ASSESSMENT CRITERIA (60%) | MARKS GIVEN BY APPRENTICE |  |  |  |  | MARKS GIVEN BY COACH |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 1-2 | 3-4 | 5-6 | 7 | 0 | 1-2 | 3-4 | 5-6 | 7 |
| 1 | Maintenance tools, equipments, PPE , materials and work area are prepared |  |  |  |  |  |  |  |  |  |  |
| 2. | Brake pads/shoes cleaned according to correct technique |  |  |  |  |  |  |  |  |  |  |
| 3. | Brake pads/shoes thickness checked |  |  |  |  |  |  |  |  |  |  |
| 4. | Brake pads/shoes replaced according to procedure and manufacturer’s manual |  |  |  |  |  |  |  |  |  |  |
| 5. | Brake fluid selected correctly |  |  |  |  |  |  |  |  |  |  |
| 6. | Brake fluid replaced according to procedure and manufacturer’s manual |  |  |  |  |  |  |  |  |  |  |
| 7. | Disc brake/brake drum cleaned according to procedure and manufacturer’s manual |  |  |  |  |  |  |  |  |  |  |
| 8. | Disc brake/brake drum replaced according to procedure and manufacturer’s manual |  |  |  |  |  |  |  |  |  |  |
| 9. | Brake hose/cable replaced according to procedure and manufacturer’s manual |  |  |  |  |  |  |  |  |  |  |
| 10. | Brake system functionality checked according to manufacturer’s specification |  |  |  |  |  |  |  |  |  |  |
| 11. | Servicing activities recorded according to the correct format and submitted to the service advisor |  |  |  |  |  |  |  |  |  |  |
|  | SUBTOTAL | A1 |  |  |  |  | A2 |  |  |  |  |
|  | FULL MARKS | 77 |  |  |  |  | 77 |  |  |  |  |
| B | ATTITUDE/SAFETY/ ENVIRONMENT (20%) | MARKS GIVEN BY APPRENTICE |  |  |  |  | MARKS GIVEN BY COACH |  |  |  |  |
|  |  | 0 | 1-2 | 3-4 | 5-6 | 7 | 0 | 1-2 | 3-4 | 5-6 | 7 |
| 1 | Attitude |  |  |  |  |  |  |  |  |  |  |
| 2. | Safety |  |  |  |  |  |  |  |  |  |  |
| 3 | Environmental |  |  |  |  |  |  |  |  |  |  |
|  |  | B1 |  |  |  |  | B2 |  |  |  |  |
|  |  | 21 |  |  |  |  | 21 |  |  |  |  |
| C | EMPLOYABILITY SKILLS (SOCIAL SKILLS) (20%) | MARKS GIVEN BY APPRENTICE |  |  |  |  | MARKS GIVEN BY COACH |  |  |  |  |
|  |  | 0 | 1-2 | 3-4 | 5-6 | 7 | 0 | 1-2 | 3-4 | 5-6 | 7 |
| 1 | Communication skills |  |  |  |  |  |  |  |  |  |  |
| 2 | Conceptual skills |  |  |  |  |  |  |  |  |  |  |
| 3 | Interpersonal skills |  |  |  |  |  |  |  |  |  |  |
| 4 | Multitasking and prioritizing |  |  |  |  |  |  |  |  |  |  |
| 5 | Self-discipline |  |  |  |  |  |  |  |  |  |  |
| 6 | Teamwork |  |  |  |  |  |  |  |  |  |  |
| 7 | Learning skill |  |  |  |  |  |  |  |  |  |  |
| 8 | Leadership skill |  |  |  |  |  |  |  |  |  |  |
|  |  | C1 |  |  |  |  | C2 |  |  |  |  |
|  | FULL MARKS | 56 |  |  |  |  | 56 |  |  |  |  |

CALCULATION TABLE

|  | MARKS GIVEN BY APPRENTICE | MARKS GIVEN BY COACH | WEIGHTED MARKS GIVEN BY APPRENTICE | WEIGHTED MARKS GIVEN BY COACH |
| --- | --- | --- | --- | --- |
| ASSESSMENT CRITERIA | A1 | A2 | A1 / 77 X 60 | A2 /77X 60 |
| ATTITUDE, SAFETY & ENVIRONMENT | B1 | B2 | B1 / 21X 20 | B2 / 21 X 20 |
| EMPLOYABILITY SKILLS (SOCIAL SKILLS) | C1 | C2 | C1 / 56X 20 | C2 / 56 X 20 |
| Total |  |  | X | Y |
| Ratio of Percentage (Apprentice: Coach) |  |  | P% | Q% |
| Grand Total |  |  | (P/100 x X) + (Q/100 x Y) |  |

COMMENTS/ RECOMMENDATIONS BY COACH

_________________________			  ______________________

COACH: 					              APPRENTICE:

DATE:							  DATE: