| LEARN AND WORK ASIGNMENT (LWA) |  |  |  |
| --- | --- | --- | --- |
| NOSS (CODE NOSS) | MOTORCYCLE SERVICING AND MAINTENANCE (TP-118-2;2012) |  |  |
| Competency Unit Title (CU CODE) | MULTI CYLINDER ENGINE OVERHAUL (ETM-XXX-05) | LEVEL | 2 |
| Competency Unit Descriptor | Multi cylinder engine overhaul competency is a set of skills to carry out multi cylinder engine overhaul that includes knowledge about various types of clutch servicing, oil pan and oil pump servicing, crankcase servicing and transmission servicing. The person whom is competent in multi cylinder engine overhaul works must be able to perform multiple cylinder engine overhaul requirement, perform clutch servicing, perform shift shaft servicing, perform oil pan and oil pump servicing, perform crankcase servicing and transmission servicing according to manufacturer’s specification, regulation requirement, vehicle specification, safety requirement and Standard Operating Procedures (SOP) |  |  |
| Candidate Name |  |  |  |
| Candidate I/C Number |  |  |  |
| Company’s Name |  |  |  |

CU WORK ACTIVITY STATEMENT:

PERFORM MULTIPLE CYLINDER ENGINE OVERHAUL ACTIVITIES

DURATION:     82 HOURS

- SETTING GOAL *:

You are required to perform multiple cylinder engine overhaul activities based on performance criteria below:

  - Service single cylinder engine overhaul manual interpreted

  - Single cylinder engine overhaul tools and equipment Selected

  - Types of clutch servicing determined

  - Tools and equipment used for clutch servicing selected

  - Servicing clutch according to procedure

  - Servicing component clutch according to service manual

  - Types of oil pan and oil pump servicing determined

  - Oiled pan and oiled pump servicing according to procedure

  - Oiled pan and oiled pump servicing according to service manual

  - Types of crankcase servicing determined

  - Servicing crank cased according to procedure

  - Servicing crank cased according to service manual

  - Types of transmission servicing determined

  - Servicing trans missed according to procedure

  - Servicing trans missed according to service manual

- PLANNING

You are required to plan activities to achieve listed setting goal of performing multiple cylinder engine overhaul activities by using resources listed below:

  - Identify  tools, materials and equipment for performing multiple cylinder engine overhaul according to listed below:

| ITEMS | RATIO (TEM : Trainees) |
| --- | --- |
| - Piston Assembly - Piston ring - Piston pin - Crankshaft assembly - Connecting rod & bearing - Big end bearing - Crank pin bearing - Main bearing - Timing sprocket - Magneto key - Oil seal - Primary clutch assembly - Friction plate - Clutch plate - Clutch housing - Clutch hub - Clutch spring - Pressure plate - Release bearing - Collar clutch - Clutch bush - Spacer - Retainer - Release complete clutch - Holder clutch - Primary gear - Wheel clutch - Bearing - Clutch release bolt - Oil ring - Upset bolt - Secondary clutch assembly - Clutch hub assembly - One way clutch - Collar - Clutch housing complete - Snap ring - Clutch spring holder - Clutch holder - Cylinder head assembly - Valve seal - Valve key/cotter/collet - Valve spring retainer/shim - Valve tappet adjusting screw - Camshaft assembly - Rocker arm - Rocker shaft - Stopper - Valve spring - Intake and exhaust valve - Etc. - Cylinder block - Oil pump gear assembly - Oil strainer - Water pump assembly - Tools - Measuring tools - Hand tools - Special tools - Overhaul gasket - Transmission gear assembly - Air gun / compressor - Lubricant - Tray - Oil can - Industrial tissue - Cleaning solvent - Scrapper - Gasket sealant - Personal Protective Equipment(PPE) | 1 : 1 1 : 1 1 : 1 1 : 1 1 : 1 1 : 1 1 : 1 1 : 1 1 : 1 1 : 1 1 : 5 1 : 1 1 : 5 1 : 1 1 : 1 1 : 5 As per required 1 : 1 1 : 5 As per required As per required 1 : 1 1 : 1 1 : 1 |

  - Refer to references below as a guidance to perform this activity.

REFERENCES

- Mark Zimmerman. (2004).The essential guide to motorcycle maintenance: Tips & Techniques To Keep Your Motorcycle In Top Condition. ISBN1884313418, 9781884313417. Whitehorse Press Series. - Dave Tharp. (1991)Trelland’s Motorcycle Reference Guide – Vol. 1. ISBN 1-895542-00-6. Trelland Industries Ltd - de Rome, L., Stanford, G. & Wood, B. (2007), Positioned for Safety 2010: A Motorcycle Safety Strategic Plan 2007-2010, Motorcycle Council of NSW, Inc., - Gibson T. & E. Benetatos (2000), Motorcycles and crash barriers prepared by Human Impact Engineering for the Barriers to Safety campaign, NSW Motorcycle Council, Sydney - Hurt, H.H. Jr., Ouellet, J.V. & Thom, D.R. (1981), Motorcycle Accident Cause Factors and Identification of Countermeasures, Final Report to National Highway Traffic Safety Administration, US Department of Transportation, PB 81-206443, 81-206450

  - List out any references related  when performing this activity such as:

    - Standard Operating Manual (SOP)/ Service manual / Manufacturer’s manual

    - Service job sheet

    - Owner’s manual

    - Etc.

C. DECISION MAKING

You are required to get coach approval before performing multiple cylinder engine overhauls.

- EXECUTE & MONITORING

You are required to perform multiple cylinder engine overhaul according to steps below:

  - Prepare multiple cylinder engine overhaul requirements

    - Obtain work order form SA

    - Determine type of engine

    - Interpret service manual multiple cylinder engine overhaul manual

    - Prepare multiple cylinder engine overhaul tools and equipments

  - Park motorcycle at work area by using main stand/safety jack

  - Disconnect terminal battery and all wire connector  from engine assembly

  - Drain out engine oil /coolant engine

  - Dismantle all related parts systematically as per service manual such as:

  - Body cover

  - Footrest

  - Exhaust assembly

  - Carburetor

  - Breather hose

  - Drive chain

  - Gear lever

  - Etc.

    - Remove the engine assembly according to service manual

    - Support chassis with a proper engine jack

    - Lift up the engine assembly and put into the chassis

  - Dismantle  engine assembly from motorcycle

  - Remove magneto cover and components

  - Remove multiple cylinder head assembly

  - Remove engine from motorcycle

  - Perform clutch servicing according to service manual

    - Dismantle clutch cable according

    - Dismantle clutch cover with proper tools

    - Take out clutch assemble, primary driven gear and clutch bush

    - Check clutch system components for faulty ( visual check )

- Check clutch plate thickness

- Measure clutch plate warpage and clutch spring free length with proper tools

    - Replace faulty clutch components according to procedure service manual

  - Dismantle LH and RH crankcase

  - Remove transmission gear and crank shaft assembly

  - Clean and check all engine parts

  - Check engine for faulty parts

    - Engine assembly

- Piston assembly

- Cylinder bore

- Head assembly

- Clutch assembly

- Magneto assembly

- Crankshaft assembly

- Transmission gear

- Gear shifting mechanism

- Connecting rod assembly

- Oil pump assembly

- Water pump assembly

- Timing chain component

- Etc

  - Perform  all engine assembly part servicing according to service manual such as:

    - Carry out measuring inspection (v block/veneer caliper/bore gauge/feeler gauge)

    - Carry out runout  test(Dial gauge assembly/surface plate)

    - Carry out connecting rod bearing journal(Plastic gauge)

  - Replace faulty part for engine assembly

  - Perform engine assembly installation according to service manual

    - Install transmission gear components

    - Install clutch components

    - Install magneto and  components

    - Install engine block assembly

    - Install engine cylinder head assembly

    - Set valve timing and valve tappet clearance adjustment according to service manual

    - Install all related cover and timing chain tensioner

    - Fix new oil seal to engine assembly

    - Fill in engine oil according to service manual

  - Perform engine assembly installation (to the motorcycle)

    - Support to chassis with a proper engine jack

    - Lift up the engine assembly and put into the chassis

    - Tighten the engine mounting bolt according to service manual

    - Install all related parts systematically as per service manual such as:

  - Exhaust assembly

  - Footrest

  - Carburetor

  - Drive chain assembly

  - Gear lever

  - Connect all wire connector  and breather hose

  - Reinstall all related cable

  - Reinstall body cover

  - Etc.

  - Perform pre start check for :

    - Spark plug condition.

    - Engine oil level

    - Coolant level

    - Level of fuel

  - Perform multiple cylinder engine functionality check according to service manual

    - Carry out compression test

    - Check gear shifting operation, clutch adjustment

    - Carry out carburetor tuning

    - Carry out visual inspection(for leakage), engine abnormal sound

  - Inform SA that motorcycle ready for test ride

    - Carry out test ride on serviced motorcycle

  - Check multiple cylinder engine performance according to service manual

  - Record multiple cylinder engine overhaul

    - Record servicing activities

    - Submit record to superior

3.14	Comply with attitude, safety and environment listed below when performing this activity

| Attitude | - Peruse the manufacturer’s specification and maintenance manual - Adhere to workshop safety practice - Meticulous in preparing report |
| --- | --- |
| Safety | - Wear appropriate PPE - Handle wiring tools, equipment and testing tools with care - Ensure tools and equipment in good working condition |
| Environment | - Make sure service area free from dust and oil |

3.15	Apply core abilities listed below when performing this activity

| Social Skills | Core Abilities |
| --- | --- |
| Communication skills | 02.03 Communicate clearly |
| Conceptual skills | 01.01 Identify and gather information 02.01 Interpret and follow manuals, instructions and SOP’s |
| Interpersonal skills | 03.06 Respond appropriately to people and situations |
| Multitasking and prioritizing | 01.02 Document information, procedures or processes 02.04 Prepare brief reports and checklists using standard form |
| Self-discipline | 03.03 Accept responsibility for own work and work area 03.02 Demonstrate integrity and apply ethical practices |
| Teamwork | 03.01 Apply cultural requirements for the workplace |
| Learning skill | 06.01 Understand systems |
| Leadership skill | 03.06 Respond appropriately to people and situations 03.03 Accept responsibility for own work and work area |

- EVALUATING

You are required to evaluate multiple cylinder engine overhaul activities using checklist below.

| A | ASSESSMENT CRITERIA (60%) | MARKS GIVEN BY APPRENTICE |  |  |  |  | MARKS GIVEN BY COACH |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 1-2 | 3-4 | 5-6 | 7 | 0 | 1-2 | 3-4 | 5-6 | 7 |
| 1 | Multiple cylinder engine overhaul tools, equipments and workplace are prepared. |  |  |  |  |  |  |  |  |  |  |
| 2. | Clutch servicing according to service manual. |  |  |  |  |  |  |  |  |  |  |
| 3. | Shift shaft servicing according to service manual. |  |  |  |  |  |  |  |  |  |  |
| 4. | Oil pump servicing according to service manual |  |  |  |  |  |  |  |  |  |  |
| 5. | Crankcase servicing according to service manual |  |  |  |  |  |  |  |  |  |  |
| 6. | Transmission servicing according to service manual |  |  |  |  |  |  |  |  |  |  |
| 7. | Multiple cylinder engine overhaul activities recorded according to correct format |  |  |  |  |  |  |  |  |  |  |
|  | SUBTOTAL | A1 |  |  |  |  | A2 |  |  |  |  |
|  | FULL MARKS | 49 |  |  |  |  | 49 |  |  |  |  |
| B | ATTITUDE/SAFETY/ ENVIRONMENT (20%) | MARKS GIVEN BY APPRENTICE |  |  |  |  | MARKS GIVEN BY COACH |  |  |  |  |
|  |  | 0 | 1-2 | 3-4 | 5-6 | 7 | 0 | 1-2 | 3-4 | 5-6 | 7 |
| 1 | Attitude |  |  |  |  |  |  |  |  |  |  |
| 2. | Safety |  |  |  |  |  |  |  |  |  |  |
| 3 | Environmental |  |  |  |  |  |  |  |  |  |  |
|  |  | B1 |  |  |  |  | B2 |  |  |  |  |
|  |  | 21 |  |  |  |  | 21 |  |  |  |  |
| C | EMPLOYABILITY SKILLS (SOCIAL SKILLS) (20%) | MARKS GIVEN BY APPRENTICE |  |  |  |  | MARKS GIVEN BY COACH |  |  |  |  |
|  |  | 0 | 1-2 | 3-4 | 5-6 | 7 | 0 | 1-2 | 3-4 | 5-6 | 7 |
| 1 | Communication skills |  |  |  |  |  |  |  |  |  |  |
| 2 | Conceptual skills |  |  |  |  |  |  |  |  |  |  |
| 3 | Interpersonal skills |  |  |  |  |  |  |  |  |  |  |
| 4 | Multitasking and prioritizing |  |  |  |  |  |  |  |  |  |  |
| 5 | Self-discipline |  |  |  |  |  |  |  |  |  |  |
| 6 | Teamwork |  |  |  |  |  |  |  |  |  |  |
| 7 | Learning skill |  |  |  |  |  |  |  |  |  |  |
| 8 | Leadership skill |  |  |  |  |  |  |  |  |  |  |
|  |  | C1 |  |  |  |  | C2 |  |  |  |  |
|  | FULL MARKS | 56 |  |  |  |  | 56 |  |  |  |  |

CALCULATION TABLE

|  | MARKS GIVEN BY APPRENTICE | MARKS GIVEN BY COACH | WEIGHTED MARKS GIVEN BY APPRENTICE | WEIGHTED MARKS GIVEN BY COACH |
| --- | --- | --- | --- | --- |
| ASSESSMENT CRITERIA | A1 | A2 | A1 / 49 X 60 | A2 / 49X 60 |
| ATTITUDE,SAFETY & ENVIRONMENT | B1 | B2 | B1 / 21X 20 | B2 / 21 X 20 |
| EMPLOYABILITY SKILLS (SOCIAL SKILLS) | C1 | C2 | C1 / 56 X 20 | C2 / 56 X 20 |
| Total |  |  | X | Y |
| Ratio of Percentage (Apprentice: Coach) |  |  | 20% | 80% |
| Grand Total |  |  | (P/100 x X) + (Q/100 x Y) |  |

COMMENTS/ RECOMMENDATIONS BY COACH

_________________________			  ______________________

COACH: 					              APPRENTICE:

DATE:							  DATE: